IP Configure tool -Manual

Version 2.5

2013-12-23

1. Start password setting

The password for the first time to start the IP configure tool is :123456

Password can be changed and its path is: Menu “Setting(S)”->”Option(O)”

2. Configure by Scan mode

2.1. Scan terminal

2.1.1. Fast scan

Fast scan all the terminals which are in the local network segment

2.1.2. Specified scan

Suitable for local network segment or across network segment terminals which their IP addresses are known  	Scan interval: Set the time from issuing scan request by IP configure tool to getting respond from terminals, the variable range is from 1 to 5000 milliseconds (1 second = 1000 ms).

Scan range: To set the scanning range from the beginning to the end of all the IP terminals within the IP range

2.2. Configure terminal

To configure parameters for terminals is available only on condition that they were be scanned

Place a tick to the corresponding parameter you want to modify and revise value then click “Modify”button. “Successful config of the terminal : 1 ” will be shown on the software If you succeed in configuring it.

3. Mandatory config

Part of the terminal does not support the scanning mode configuration, it can be configured with a Mandatory config.

All terminals support "Mandatory config", but you have to be sure that multicast packets can reach one terminal only in the network, otherwise, All terminals in the network will be configured to the same parameters which result in problems(e.g : All terminals can not log on server duo to IP conflict)

Security Check: When you use “Mandatory config” and have pressed “Send parameters” button already , The config tool won’t send parameters to terminal immediately, it will quickly scan all terminals in this local network segment firstly, if it finds more than one terminal, it will stop sending parameters

NOTE: The config tool can not check across network segment terminals, If multicast packets can reach across network segment terminals, All terminals in local network segment and across network segment will be modified same parameters, so we suggest you to unplug the outside network cable before setting

To configure parameters in this manner is only responsible for sending out the parameters and will not check whether the terminal receives the packet or not, we have the following ways to confirm it:

a. To see the parameters on the LCD display( part of the terminal)

b. Part of the terminal will restart after the success of setting and with the "click" sound of relay

c. To see whether the corresponding terminals have logged on server or not

4. Upgraded firmware

Note: When the terminal is broken or has to be added new features, You’d better contact the manufacturers before you carry out updating as unnormal operations will result in possible damage to the terminal hardware circuit.

Operation steps:

a. Scan terminals.

b. Select one or more devices in the terminal list (press and hold shift key for continuous selection, press and hold ctrl key for jump choice)

c. Click the menu bar "firmware" -> "Update", Firmware Update dialog box will be popped up, click the button on the right of "Source File" to select an updated firmware file.

d timeout setting: Updating will give up if the terminal does not respond over the time (default 10000 milliseconds).

e. Click “start update” to enter the update status, The config tool will display the numbers of upgraded terminals when the updating is finished

5. Trouble shooting of terminals can not be scanned

a. Turn off Windows Firewall and exit anti-virus software.

b. In some cases, scan data can not reach terminals because of crossing network segment and router, please scan terminal under the same network segment or use web page to configure terminals (Only part of terminals support web configuration).

c. Specify time of "Scan interval in more than 500